AIR VENTS

1” Air Vacuum Relief Valve Specification

The air vacuum relief valve provides instant and continuous vacuum relief and non-continuous air relief.  Both the body and the removable dirt cover shall be constructed of molded plastic. The body and the dirt cover shall be connected with a 3/4 inch hose thread.    The ball shall be constructed of low density plastic and the internal seat shall be constructed of vinyl.  The air vacuum relief valve shall seal at 5 psi.  Inlet size shall be a 1 inch male pipe thread.  The air vent shall be E-Z Set item number AVBK-1.

Note:  Maximum flow of 50 gpm per 1” air vacuum relief.

PRESSURE REGULATORS

| ITEM NUMBER | OUTLET PRESSURE | FLOW RANGE | MAX. INLET PRESSURES | INLET SIZE | OUTLET SIZE |
| --- | --- | --- | --- | --- | --- |
| PR-20MF | 20 psi | 2 - 20 gpm | 150 psi | 1" FIPT | 1" FIPT |
| PR-20HF | 20 psi | 10 - 32 gpm | 100 psi | 1.25" FIPT | 1" FIPT |
| PR-20XF | 20 psi | 20-90 gpm | 90 psi | 3” I.D. socket | 3” I.D. socket |
| PR-30MF | 30 psi | 2-20 gpm | 150 psi | 1" FIPT | 1" FIPT |
| PR-30HF | 30 psi | 10-32 gpm | 100 psi | 1.25" FIPT | 1" FIPT |
| PR-30XF | 30 psi | 20-90 gpm | 100 psi | 3” I.D. socket | 3” I.D. socket |
| PR-40MF | 40 psi | 2-20 pgm | 150 psi | 1" FIPT | 1" FIPT |
| PR-40HF | 40 psi | 10-32 gpm | 100 psi | 1.25" FIPT | 1" FIPT |
| PR-40XF | 40 psi | 20-90 gpm | 125 psi | 3” I.D. socket | 3” I.D. socket |
| PR-50MF | 50 psi | 2 - 20 gpm | 150 psi | 1" FIPT | 1" FIPT |
| PR-50HF | 50 psi | 10 - 32 gpm | 100 psi | 1.25" FIPT | 1" FIPT |
| PR-50XF | 50 psi | 20-90 gpm | 125 psi | 3” I.D. socket | 3” I.D. socket |

Pressure Regulator Specification

Pressure regulator shall be designed to handle steady inlet pressures of ____ psi and withstand severe water hammer extremes.  It shall handle flow rates between ___ and ___ gpm.  Flow restriction shall be negligible until the factory preset operating pressure of ___psi is reached.  Regulating accuracy shall be within +/- 6%.  Inlet size shall be ___________.  Outlet size shall be _____________.  Pressure regulator shall be constructed of high impact engineering grade thermoplastics for strength and durability.  Regulation shall be accomplished by a fixed stainless steel compression spring which shall be enclosed in a chamber separate from the normal water passage.  Each regulator shall be water tested for accuracy.  Preset pressure regulators shall be E-Z Set model number PR-______.

DRIP FITTINGS

Dripline Fittings

The lockslip fittings shall be molded of high grade plastic.  The drip tubing is pushed over a barb end, then secured with a locking nut.  The fitting has the ability to be removed and reapplied with the locking nut.

a. Lockslip Adapters.  The Lockslip adapter is used to connect the dripline to a PVC fitting.  The adapter glues into a ¾ inch slip fitting.  The lockslip adapter shall be E-Z Set Drip part number LSLIP-600.

b. Lockslip Couplings.  The coupling is used to connect two pieces of dripline together. The lockslip coupling shall be E-Z Set Drip part number LC-600.

c. Lockslip Elbows :  The elbow is used to connect dripline to dripline in a 90 degree configuration. The lockslip elbow shall be E-Z Set Drip part number LEL-600.